TASK1:

1. The Device is Completely Unresponsive :

- The Issue: The keyboard or mouse does not power on, register any inputs, or show any signs of life when plugged in.

- The Solution: Plug the device into a different USB port on the laptop (or test it on another computer) to determine if the issue is a dead USB port, a faulty cable, or a dead device.

2. Intermittent Connection (Disconnecting and Reconnecting):

- The Issue: The keyboard or mouse repeatedly disconnects and reconnects, accompanied by the system's USB plug/unplug sound.

- The Solution: Disable USB Selective Suspend in your laptop's advanced power settings to prevent the operating system from temporarily turning off power to the USB ports to save battery.

3. Laggy Mouse Cursor or Delayed Key Inputs:

- The Issue: There is a noticeable delay (latency) between moving the mouse or pressing a key and the action happening on the screen.

- The Solution: If using a wireless USB receiver (dongle), plug it into a USB 2.0 port rather than a USB 3.0 port, as active USB 3.0 ports are known to emit 2.4GHz radio frequency interference that degrades wireless peripheral performance.

4. Keys Registering Double Inputs (Key Chattering) :

- The Issue: Pressing a key once on the keyboard registers multiple identical letters on the screen (e.g., typing "h-e-l-l-o" yields “HELLO”

- The Solution: Adjust the Repeat Delay slider to "Long" and the Repeat Rate to "Slow" in your operating system's keyboard settings, or blow compressed air under the keycaps to clear out dust causing mechanical bouncing.

5. "USB Device Not Recognized" Error Windows Pop-up :

- The Issue: The laptop detects that something is plugged in but fails to load the appropriate driver, rendering the keyboard or mouse useless.

- The Solution: Open the Device Manager, expand the "Universal Serial Bus controllers" section, right-click on the malfunctioning device or USB Root Hub, select Uninstall device, and then restart your laptop to force the system to reinstall clean, default drivers.

TASK 2 :

Step 1: Check the Physical and Power Indicators

Before diving into software menus, rule out basic hardware issues:

- Action: Press and hold the power button on the headphones. Check the LED indicator light.

- What to look for: If there is no light, the battery is dead; plug them in to charge. If a red or blinking battery light appears, the charge may be too low to initiate a stable Bluetooth handshake.

Step 2: Toggle Bluetooth and Check Airplane Mode :

Sometimes the phone's wireless stack gets temporarily stuck.

- Action: Swipe down to open your phone’s Quick Settings/Control Center. Toggle Bluetooth OFF, wait 5 seconds, and toggle it back ON.

- What to look for: Ensure Airplane Mode is turned off, as this completely disables cellular and wireless radios on some devices.

Step 3: Force Pairing Mode on the Headphones :

Headphones will not connect to a new device (or reconnect if they get desynced) unless they are actively broadcasting their pairing signal.

- Action: Turn the headphones off. Then, press and hold the power button (or dedicated Bluetooth button) continuously for 5–7 seconds.

- What to look for: Do not let go when you hear the "power on" prompt. Keep holding until the LED light begins flashing rapidly (usually alternating blue and red, or flashing white). This indicates they are actively in pairing mode.

Step 4: Clear the Phone's Bluetooth Cache (Forget Device) :

If the headphones were paired previously, the phone might be trying to use an outdated or corrupted connection profile.

- Action: Go to Settings > Connected Devices / Bluetooth on your phone. Find your headphones under the "Saved Devices" list, tap the gear icon next to them, and select Forget or Unpair.

- What to look for: After forgetting the device, turn pairing mode back on your headphones (Step 3), tap Scan / Pair New Device on your phone, and see if the headphones reappear under available devices to create a fresh connection.

Step 5: Rule out "Device Hijacking" (The Multi-Device Conflict) :

Bluetooth headphones often automatically connect to the last device they were used with, blocking out your phone.

- Action: Check if your headphones have accidentally connected to your laptop, tablet, or a family member's phone nearby.

- What to look for: If the headphone LED shows a solid "connected" light but your phone says "Not Connected," turn off Bluetooth entirely on your computer or other nearby devices to break the hijack loop, then try connecting to your phone again.

Step 6: The Ultimate Reset :

If the software handshake is still failing, a hardware reset of the headphones' firmware is required.

- Action: Look up your specific model's factory reset command (usually involves holding the power and volume down buttons simultaneously for 10 seconds while plugged into a charger, or holding the button on the charging case with the buds inside).

- What to look for: The indicator light will usually flash a specific color sequence (like blinking amber 3 times) to confirm the reset. Once reset, restart your phone, and attempt the pairing process from scratch.

TASK 3:

1. Check Windows Privacy Permissions :

Windows 10 has built-in privacy toggles that can block applications from accessing your camera entirely.

- What to do: Go to Settings > Privacy > Camera. Ensure that "Camera access for this device" is turned on, and verify that the toggle for "Allow desktop apps to access your camera" (which includes Zoom) is enabled.

2. Verify Zoom's Internal Video Settings :

Sometimes the hardware is working fine, but Zoom is simply looking at the wrong input source.

- What to do: In the Zoom client, click your profile picture and go to Settings > Video. Under the Camera dropdown menu, make sure your specific external webcam model is selected instead of an integrated webcam or a virtual camera driver.

3. Check Device Manager for Driver Issues :

Windows needs to properly recognize the webcam hardware to pipe the video feed to your apps.

- What to do: Right-click the Start button and open Device Manager. Look for "Cameras" or "Imaging devices". If you see a yellow exclamation mark next to your webcam, right-click it, select Update driver, or choose Uninstall device and unplug/replug the webcam to force a clean driver reinstall.

4. Test Different USB Ports and Connections :

Webcams require decent bandwidth and power, which can fail if plugged into a bad port or an unpowered USB hub.

- What to do: Unplug the webcam and plug it directly into a different USB port on your computer (preferably a USB 3.0 port, usually color-coded blue). Avoid using external USB hubs or splitters, as they may not provide enough power to run the camera.

5. Rule out Application Conflicts :

Only one application can control a webcam's video stream at any given time.

- What to do: Close any other programs that might be secretly using the camera in the background—such as Microsoft Teams, Skype, Discord, or the Windows native "Camera" app. Restart Zoom after closing these applications.

6. Inspect Physical Hardware Controls :

Many modern external webcams feature physical built-in privacy mechanisms that users easily overlook.

- What to do: Check if your webcam has a physical privacy slider/shutter covering the lens, or a physical hardware "On/Off" toggle switch on its side. Ensure the lens is clear and the switch is turned on.

TASK 4 :

How to Run the Windows Printer Troubleshooter :

- Open the Settings Menu: Click the Start menu and select the Settings gear icon (or press Win + I).

- Navigate to Troubleshoot: Go to Update & Security > Troubleshooting (on Windows 10) or System > Troubleshoot (on Windows 11).

- Run the Tool: Click on Additional troubleshooters (or Other troubleshooters), find Printer in the list, and click Run the troubleshooter.

- Select the Printer: The tool will open a wizard and ask you to select the specific printer that is showing the offline status. Select it and click Next.

What the Information the Tool Provides During the Process :

As the troubleshooter runs, it acts like a digital detective, providing real-time feedback and information across three distinct phases:

- Diagnostic Progress (What it is checking): The tool displays a live progress bar alongside text showing exactly what subsystem it is analyzing. It will explicitly state it is "Checking the Spooler service," "Checking network connectivity," or "Verifying device drivers." This lets you know where the potential breakdown is happening.

- Root Cause Identification (What is wrong): If the tool discovers the issue, it will pause and display a clear summary of the problem. For an offline printer, it commonly informs you of things like:

."The Print Spooler service is stopped."

."The printer is not reachable over the network."

."The 'Use Printer Offline' setting is enabled."

- Actionable Resolution Choices: Instead of just telling you what's broken, the troubleshooter provides choice prompts. It will say something like, "We found a problem. Would you like us to restart the Print Spooler service to fix it?" accompanied by an "Apply this fix" button.